SMART BUOY — COMPLETE HARDWARE, SENSOR & COMPUTE BILL OF MATERIALS (BOM)

End-to-End Marine Vision (YOLOv11m), 46-Band Wave Spectra, Met-Ocean & 7-Channel Water Quality Sonde Platform

1. Purpose

This document details the complete hardware, sensor, edge AI compute, telemetry, and solar-power Bill of Materials (BOM) required to assemble and deploy the Smart Ocean Buoy platform. Each hardware subsystem maps directly to the validated software pipeline: (1) 46-band wave energy spectra and 6-DOF motion telemetry, (2) offshore meteorological monitoring, (3) 7-channel submerged water-quality sonde anomaly detection, and (4) 4-track YOLOv11m marine computer vision (navigation buoys, COLREGs vessels, Search-and-Rescue swimmers, and optical horizon attitude).

2. Hardware, Sensor & Compute Cost Breakup (Option A: NVIDIA Jetson Orin Nano Super 8GB)

| Sr. | Subsystem | Component / Sensor Specification | Qty | Basic Unit (Excl. GST) | Basic Total (Excl. GST) | GST @ 18% | Total Payable (Incl. GST) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Edge AI Compute | NVIDIA Jetson Orin Nano Super Developer Kit (8 GB, 67 TOPS) with carrier & heatsink | 1 | ₹23,500 | ₹23,500 | ₹4,230 | ₹27,730 |
| 2 | Local Storage | 256 GB Industrial M.2 NVMe SSD (Wide-temp -40°C to +85°C) | 1 | ₹3,800 | ₹3,800 | ₹684 | ₹4,484 |
| 3 | Watchdog & Logger MCU | STM32H743 / Industrial ESP32-S3 Board + Dual Isolated RS-485, SDI-12 & 4-Ch MOSFET Switch | 1 | ₹4,500 | ₹4,500 | ₹810 | ₹5,310 |
| 4 | Wave Motion IMU | 9-Axis Marine AHRS / Wave IMU (WitMotion HWT906 IP67 + ADXL355 Low-Noise MEMS, 20-50 Hz) | 1 | ₹14,500 | ₹14,500 | ₹2,610 | ₹17,110 |
| 5 | GNSS / GPS | u-blox ZED-F9P / NEO-M9N Multi-GNSS Receiver + Active IP67 Marine Dome Antenna | 1 | ₹6,500 | ₹6,500 | ₹1,170 | ₹7,670 |
| 6 | Marine Vision Cameras | Sony Starvis 2 (IMX678 4K / IMX462 1080p) Wide-Angle (100° HFOV) IP67 Marine Camera (Hydrophobic) | 2 | ₹11,500 | ₹23,000 | ₹4,140 | ₹27,140 |
| 7 | Met-Ocean Wind | 2-Axis Ultrasonic Marine Anemometer (WSPD, GST, WDIR) — IP68 ASA Housing, RS-485 / NMEA-0183 | 1 | ₹18,500 | ₹18,500 | ₹3,330 | ₹21,830 |
| 8 | Barometer & Air Temp | Bosch BMP390L + SHT35 Barometric Pressure & Temp Probe + 6-Plate UV Gill Shield + ePTFE Vent | 1 | ₹3,200 | ₹3,200 | ₹576 | ₹3,776 |
| 9 | WQ Sonde: Cond/Sal/Temp | Submersible Titanium 4-Electrode / Inductive Conductivity, Salinity (0-42 PSU) & PT1000 Temp Sensor | 1 | ₹16,000 | ₹16,000 | ₹2,880 | ₹18,880 |
| 10 | WQ Sonde: Dissolved O2 | Submersible Optical Fluorescence Dissolved Oxygen (ODO) Sensor (0-20 mg/L, 0-200%, RS-485, IP68) | 1 | ₹28,000 | ₹28,000 | ₹5,040 | ₹33,040 |
| 11 | WQ Sonde: Turbidity | Submersible ISO 7027 90° NIR (860 nm) Turbidity Sensor (0-1000 NTU) with Self-Cleaning Wiper (RS-485) | 1 | ₹32,000 | ₹32,000 | ₹5,760 | ₹37,760 |
| 12 | WQ Sonde: Marine pH | Submersible Industrial Double-Junction Salt-Bridge Marine pH Electrode (0-14 pH, RS-485, IP68) | 1 | ₹9,500 | ₹9,500 | ₹1,710 | ₹11,210 |
| 13 | 4G LTE Telemetry | Teltonika RUT200 Industrial 4G LTE Router + High-Gain IP68 Marine Omni MIMO Antenna | 1 | ₹16,500 | ₹16,500 | ₹2,970 | ₹19,470 |
| 14 | Marine Safety | Self-Contained IALA Yellow (Fl Y 4s) Solar/12V LED Marine Lantern (3 NM) + Octahedral Radar Reflector | 1 | ₹11,000 | ₹11,000 | ₹1,980 | ₹12,980 |
| 15 | Solar Array & MPPT | 4x 25W (100W Total) Marine ETFE Monocrystalline Solar Panels + Victron SmartSolar MPPT 75/15 | 1 | ₹15,500 | ₹15,500 | ₹2,790 | ₹18,290 |
| 16 | Battery Bank | 12.8V 100Ah (1,280 Wh) LiFePO4 Marine Deep-Cycle Battery Pack with 100A Smart BMS | 1 | ₹22,000 | ₹22,000 | ₹3,960 | ₹25,960 |
| 17 | Marine Connectors & Rail | IP68 M12 Bulkhead Connectors (8x), Isolated 12V->19V/5V DC-DC, TVS Surge Protection & 316L Sonde Cage | 1 | ₹14,000 | ₹14,000 | ₹2,520 | ₹16,520 |
|  | TOTAL (Option A) | Complete Smart Buoy Hardware Stack (Jetson Orin Nano Super 8GB) |  |  | ₹2,62,000 | ₹47,160 | ₹3,09,160 |

3. Alternative Low-Power Compute Option B (Raspberry Pi 5 8GB + Hailo-8 26 TOPS AI HAT+)

Replacing Item #1 (NVIDIA Jetson Orin Nano Super 8GB: Basic ₹23,500 + GST ₹4,230 = ₹27,730 Payable) with Raspberry Pi 5 (8 GB) + Official Raspberry Pi AI HAT+ (26 TOPS Hailo-8 NPU) + Active Cooler + PCIe M.2 Base (Basic ₹17,500 + GST ₹3,150 = ₹20,650 Payable) yields a total system cost of:
• Basic Total (Excl. GST): ₹2,56,000
• GST @ 18%: ₹46,080
• Total Payable (Incl. 18% GST): ₹3,02,080

4. Notes & Billing Caveats

All figures in the breakup table explicitly separate GST-exclusive Basic prices from 18% GST and GST-inclusive Payable totals.

All submerged water-quality sensors (Items 9–12) use isolated RS-485 Modbus RTU signalling and should be mounted inside a perforated 316L stainless-steel / copper anti-fouling guard beneath the buoy float.

Under duty-cycled operation (STM32/ESP32 logging IMU at 20 Hz continuously while waking the AI compute board for 45 seconds every 10 minutes), average system power draw is 2.40 W (57.6 Wh/day), providing 17.7 days of battery autonomy on a single charge without sunlight.